FINAL PROJECT REPORT

DEPARTMENT OF ELECTRICAL ENGINEERING

IIT KANPUR

NAME: DHRUV SARASWAT

Institute: NIT Rourkela (B.Tech, EE)

PF. NO: C20408

PROJECT NO: IUSSTF/EE/2017282D

PROJECT TITLE: Ui- Assist:IIT Kanpur Pilot 2B

Principal Investigator: Prof. Ankush Sharma

Gateway and STM32 (blue pill and black pill)                                                         controlled board

Altium Tutorial: https://youtu.be/PqFtSpAXB9Q

This project based on designing of a printed circuit board containing STM32 blue pill, Lora gateway, DB9 male connector, Hi-Link(AC-DC convertor), UFL antenna to transmit signals and IC's like MAX485 and MAX232 with their connections with several capacitors, resistors , STM32 (blue pill) and various other components on the board. The dimension of the PCB is 7.5mm x 4.3mm. this PCB helps in collecting the data from a meter like Voltage, Current, Power (KWh) and frequency and send it in rc packets form to the receiver consisting of Raspberry Pi and LoRaWAN concentrator which are connected through a gateway. The PCB model of that gateway is also designed by me.

Gateway to connect Raspberry PI and LoRaWAN concentrator

Project name: Pi Hat

PCB document-2d

3d front side:

Schematic:

3D backside

PCB Lib

SCH Lib

Bottom layer

Distance between 2 pins of all 4 headers and switch = 2.54mm

Standoff height for 20x1 male header= 100 mil

Standoff height for 20x2 male header and 3x1 header= 00 mil

Standoff height for switch= 1.346mm

Links

3x1 header

2-644861-3 TE Connectivity AMP Connectors | Connectors, Interconnects | DigiKey

10x1 header

0705430114 Molex | Connectors, Interconnects | DigiKey

20x1 header

PPTC202LFBN-RC footprint & symbol by Sullins Connector Solutions | SnapEDA

Switch

MS12ANW03 NKK Switches | Switches | DigiKey

How to start project:

- File> new> project> name it.

- Right click on project> add new to project> add pcb, schematic, pcblib and sch lib.

- After adding all 4 of them save it.

How to add footprint and 3d body

- Search the respective product in digikey and filter the ead/cad file.

- Download the cad model and 3d body from link of snapeda or ultra librarian and extract the zip file.

- Open Altium and drag files from download directly to Altium.

- Then go to pcblib>place>3d body>select relevant step file>okay.

- View> 3d layout mode / press 3 > check the orientation of 3d body and place accordingly. Use enter to rotate it.

- Else right click > properties>change rotation angle and standoff height. Fit the 3d body on footprint correctly.

- Press 2 to go to 2d view and save.

- Repeat the same process for adding different components.

- Save the pcblib and schlib after adding all the components.

- For models downloaded from ultra librarian to add them

- File>run script>browse> from file> select extracted file> okay

- File>select text file> start import.

How to add components in schematic and do the connections:

- Go to schematic > panels > suitable schlib> components> drag all the components one by one.

- For doing connections either use wire or net label

- Put net label on one end and rename accordingly

- Use same net label on other end where u need to connect.

How to place the components from schematic to  pcb:

- Schematic> design> update pcb doc> validate changes> execute change.

- Place the components on the board so that they occupy least space

Resize the pcb board:

- Select board out, place the origin on bottom left corner.

- Edit> origin> set> place on corner.

- Place line> press J then L> select the location and define the board.

- Use J then L 5 times. To return to origin again

- Click on one of the line > press tab to select all lines> design > board shape> define board shape.

How to do routing between components:

- Interactive routing> select the terminals and join.

- If required do the routing using Via.

- Place via > on one/both terminals as needed and do the routing on back layer

- After placement and routing of components>tools> design rule check.

- Then output job file

- Right click on project>add new to project> output job file

- Fabrication output>gerber file > pcb doc.

- Similarly obtain nc drill file

STM32F103 (BLUE PILL) controlled PCB

2D Top Layer

2d Bottom Layer

3D Top Layer

3D Bottom Layer

Schematic

PCB Lib:

SCH Lib:

Links:

MAX485

MAX485ESA+ footprint & symbol by Analog Devices | SnapEDA

10K RESISTOR

RK73H2BTTD1002F KOA Speer Electronics, Inc. | Resistors | DigiKey

0.1uF capacitor

GRM033C80J104KE15D Murata Electronics | Capacitors | DigiKey

10uF capacitor

CL10A106MQ8NNNC Samsung Electro-Mechanics | Capacitors | DigiKey

20k resistor

RC1206FR-0720KL footprint & symbol by Yageo | SnapEDA

120ohm resistor

AC1206FR-07120RL YAGEO | Resistors | DigiKey

1k resistor

RC1206FR-071KL YAGEO | Resistors | DigiKey

led

5988270107F Dialight | Optoelectronics | DigiKey

UFL

U.FL-R-SMT(01) Hirose Electric Co Ltd | Connectors, Interconnects | DigiKey

DE09

DE09-PL-25 Adam Tech | Connectors, Interconnects | DigiKey

RFM 95

RFM95W-868S2 RF Solutions | RF and Wireless | DigiKey

1Uf capacitor

CL31B105KBHNNNF Samsung Electro-Mechanics | Capacitors | DigiKey

HI LINK

Buy HLK 2M05 5V 2W AC DC SMPS Power Module | Robu.in

STM 232

ST232CDR STMicroelectronics | Integrated Circuits (ICs) | DigiKey

PHOENIX CONTACT

1803277 Phoenix Contact | Connectors, Interconnects | DigiKey